Lab Assignment: Exploring the basics of NAT

Topology Overview:

- Firewall Machine

  - Loopback: Standard loopback interface.

  - LAN: Static IP 10.81.100.1/24 on eth0.

  - Outside: Static IP 200.1.1.1/24 on eth1.

  - DMZ: Static IP 172.16.1.1/24 on eth2.

  - Cloud: Static IP 192.168.100.1/24 on eth3.

- Outside Machine

  - Loopback: Standard loopback interface.

  - LAN: Static IP 200.1.1.9/24 on eth0.

  - Default Route: Gateway 200.1.1.1.

- DMZ Machine

  - Loopback: Standard loopback interface.

  - LAN: Static IP 172.16.1.8/24 on eth0.

  - Default Route: Gateway 172.16.1.1.

- Inside1 Machine

  - Loopback: Standard loopback interface.

  - LAN: Static IP 10.81.100.10/24 on eth0.

  - Default Route: Gateway 10.81.100.1.

- Inside2 Machine

  - Loopback: Standard loopback interface.

  - LAN: Static IP 10.81.100.11/24 on eth0.

  - Default Route: Gateway 10.81.100.1.

Additional Resources for Testing:

For testing firewall access, you can utilize full-blown Linux computers available in GNS3. To access them, select "Computer" in GNS3, then drag any of the computers titled: Inside1, Inside2, DMZ, etc., onto your topology. These machines have full services enabled for testing, such as SSH, web, and they respond to pings.

Part 1: Basic Firewall Setup

- View the current firewall rules on the Firewall Machine:

root@Firewall:~# iptables –L

- Are there any pre-defined rules?

- What is the default policy for the INPUT, FORWARD, and OUTPUT chains?

- Set a default policy on the INPUT chain to drop packets:

root@Firewall:~# iptables –P INPUT DROP

- Based on this rule, what PINGs from Inside1 Machine should be successful? Which PINGs will fail?

- From Inside1 Machine, can the web service be accessed on the Outside Machine?

- Add a rule to drop packets destined to other networks:

root@Firewall:~# iptables -P FORWARD DROP

- Based on this rule, what PINGs from Inside1 Machine should be successful? Which PINGs will fail?

- From Inside1 Machine, can the web service be accessed on the Outside Machine?

Part 2: Stateful Firewall Configuration

- Allow all packets out from the LAN interface (eth0) to the Outside interface (eth1):

root@Firewall:~# iptables –A FORWARD –i eth0 –o eth1 –j ACCEPT

- Send a PING from Inside1 Machine to the Outside Machine. Is the PING successful?

- View the firewall rules. Are there any packets matching the rule you just made?

- Allow return packets that match the outbound packets:

root@Firewall:~# iptables –A FORWARD –i eth1 –o eth0 –m conntrack --ctstate ESTABLISHED,RELATED –j ACCEPT

- Send a PING from Inside1 Machine to the Outside Machine again. Is the PING successful?

- View the firewall rules again. Are there matches for both rules you created?

- Test telnet or web from Inside1 Machine to the Outside Machine. Is it successful?

Part 3: NAT Configuration

- Set the iptables rules for NAT on the Firewall Machine:

root@Firewall:~# iptables –t nat -A POSTROUTING –o eth1 –j MASQUERADE

- Confirm your configuration:

root@Firewall:~# iptables –L –v -t nat

- From Inside1 Machine, browse to the Outside Machine's IP address. Is the web service accessible?

Part 4: DNAT Configuration

- Set up DNAT for packets coming from the Outside Machine to the Inside1 Machine's web service:

root@Firewall:~# iptables –t nat –A PREROUTING –i eth1 –p tcp --dport 80 –j DNAT --to-destination 10.81.100.10:80

- From the Outside Machine, open the web service running on Inside1 Machine using the public IP 200.1.1.1 assigned to the Firewall Machine's outside interface. Is the web service accessible?

- Allow packets through the FORWARD chain in the FILTER table:

root@Firewall:~# iptables –A FORWARD –p tcp –d 10.81.100.10 --dport 80 –j ACCEPT

- Test accessing the web service again from the Outside Machine. Is it successful?

Conclusion:

After completing the lab, verify your configurations by testing the traffic flows. Ensure that you've met all the requirements and that other traffic flows remain unaffected. Remember, the key is to apply the knowledge you've learned about iptables and think critically about the desired outcomes. Good luck!