## 🧭 Objective

Learn how to:

- Capture DHCP (DORA), ARP, and DNS traffic

- Understand basic network settings (IP, subnet mask, gateway, DNS)

- Set a static IP safely

- Explore how networks behave using Ping, ARP, and Wireshark

- Understand basic TCP vs UDP differences

# 🚨 Important Setup First: Set Your VM to Bridged Mode (Correctly)

Before starting the lab, you must put your virtual machine’s network adapter into Bridged Mode and check the physical adapter binding.

### 🛠 How to Set Bridged Mode Correctly (VMware Workstation)

- Power off your virtual machine.

- In VMware Workstation, open your VM settings.

- Select Network Adapter.

- Change the connection from NAT to Bridged: Connected directly to the physical network.

- Important: Open the Virtual Network Editor:

  - Make sure Bridged is bound to your real, active network adapter (Ethernet or Wi-Fi).

  - Do not leave it on "Automatic."

  - Manually select your real network card.

- Save the settings.

- Power your VM back on.

### 📢 Why This Matters

- "Automatic" often picks the wrong adapter.

- If you bridge incorrectly, your VM will not get a real IP or see other devices.

- Correct binding makes your VM behave like a real device on the local network.

# Part 1: Capture DHCP and ARP Traffic

## 🚪 Section A: Capture DORA (DHCP Discovery Process)

### What is DORA?

- Discover

- Offer

- Request

- Acknowledge

It’s how your computer asks the network for an IP address automatically.

### Steps:

- Open Wireshark.

- Start a packet capture on your active network adapter.

- In Windows, disable your network adapter:

  - Control Panel → Network and Sharing Center → Change adapter settings

  - Right-click your adapter → Disable

- Wait 10 seconds, then re-enable the adapter.

- Watch Wireshark for DHCP packets.

Helpful filter to use:

bootp

Note: DHCP was built on top of an older protocol called BOOTP.
 Wireshark still labels DHCP traffic under the bootp filter because of this shared history.

### ✅ Checkpoint:

- Find all four parts of DORA.

- 👉 Call me (Steve) over when you find DORA to get your checkmark.

## 📡 Section B: Capture ARP (Address Resolution Protocol)

### What is ARP?

- ARP lets computers find out each other's MAC addresses on the same network.

### Steps:

- Keep Wireshark running.

In Command Prompt, ping any IP address you haven't pinged yet. Example:

 ping 192.168.1.200

- Watch Wireshark for ARP packets.

Helpful filter to use:

arp

### ✅ Checkpoint:

- Find the ARP request and response.

- 👉 Call me (Steve) over when you capture ARP.

# Part 2: Explore Your Network Basics and Set a Static IP

## 🖥️ Section A: Find Your Network Information

- Open Command Prompt.

Run:

 ipconfig /all

- Find and write down:

  - IPv4 Address

  - Subnet Mask

  - Default Gateway

  - DNS Server(s)

### 🧠 Why It Matters

- IP Address: Your device’s "street address."

- Subnet Mask: Defines your "neighborhood."

- Default Gateway: The "door" to leave your local network.

- DNS Server: Turns names like google.com into IP addresses.

## 🛠️ Section B: Pick a Static IP Address

Before setting anything:

- Choose a nearby unused IP address (example: 192.168.1.250).

## 🔍 Section C: Manually Check for IP Conflicts

In Command Prompt, ping the address you want to use:

ping 192.168.1.250

Watch Command Prompt and Wireshark at the same time.

Apply this filter in Wireshark:

arp or icmp

### 🔎 There are Three Possible Outcomes:

| Situation | Ping Result | Wireshark Shows | Meaning |
| --- | --- | --- | --- |
| 1. Successful Ping | Replies (time=XXms) | ARP + ICMP Echo-Reply | IP is in use. Don't use it. |
| 2. Request Timed Out, ARP Seen | No ping replies | ARP request + ARP reply | Device owns the IP but blocks pings. Still in use. Don't use it. |
| 3. Destination Host Unreachable | Immediate error | No ARP reply, no ICMP | IP is free and safe to use. |

### 🧠 Why This Matters

You are manually performing Duplicate Address Detection (DAD), like IPv6 does automatically.

## 🛠️ Section D: Set Your Static IP

- Go to:

  - Control Panel → Network and Sharing Center → Change adapter settings

- Right-click your adapter → Properties.

- Select Internet Protocol Version 4 (TCP/IPv4) → Click Properties.

- Set:

  - Your new static IP (confirmed free)

  - Same Subnet Mask

  - Same Default Gateway

  - Same DNS Server(s)

### ✅ Final Checkpoint:

- Ping your gateway to confirm network access.

- Open a browser and visit a website.

- 👉 Call me (Steve) over once your static IP is working.

# Part 3: Capture DNS Traffic

## 🌐 Section A: Capture a DNS Query

DNS (Domain Name System) translates human names into IP addresses.

### Steps:

- Keep Wireshark running.

In Command Prompt, type:

 nslookup example.com

- (Pick any domain you like.)

- Watch Wireshark for DNS traffic.

Helpful filter to use:

dns

### ✅ Checkpoint:

- Capture a DNS request and response.

- 👉 Call me (Steve) over when you have DNS captured.

# 🚀 Optional Challenge (Not Graded)

## 🔍 Explore TCP vs UDP in Wireshark

- Look at your captured traffic:

  - DHCP uses UDP

  - DNS usually uses UDP (sometimes TCP for big answers)

  - HTTP uses TCP

Think about:

- UDP is faster but unreliable (no handshake).

- TCP is slower but reliable (guarantees delivery and ordering).

Why?

- DHCP and DNS are quick and simple (UDP).

- HTTP needs guaranteed, ordered delivery (TCP).

# 🎯 Final Notes

Today you:

- Captured DHCP, ARP, and DNS traffic

- Explored your network’s basics

- Set a static IP properly

- Learned how to detect IP conflicts manually

- Saw real-world TCP and UDP in action