## ✅ Section 1: BIOS Password Handling

### If your machine already has a BIOS/System password, clear it using the jumper method below. Then set your own, verify, and clear it again.

### Step 1: Clear Existing BIOS Password

- Find the PSWD jumper on the motherboard.

- Move the 2-pin jumper from pins 2–3 to pins 1–2.

- Power on, wait for the screen to POST, then power off.

- Return the jumper to its original position (pins 2–3).

- Call Steve over to verify the password is cleared.

### Step 2: Set a BIOS Password

- Boot into BIOS (F2 at startup).

- Set a Supervisor or System password.

- Call Steve over to confirm.

### Step 3: Clear the Password

- Go back into BIOS.

- Remove the password (enter current one, leave new field blank).

- Call Steve over one last time to verify.

## 🔍 Section 2: BIOS Scavenger Hunt

Explore BIOS settings and find the following values or options. Write them down.
 When you’re finished:

- Call Steve to check your answers.

- Once your sheet is approved, you must check the answers of one other student.

  - Put your initials on their sheet when you verify it.

### BIOS Items to Locate:

- BIOS version and date

- CPU model and speed

- Total system RAM

- Boot sequence order

- TPM enabled or disabled?

- Virtualization (VT-x or AMD-V) on/off?

- Secure Boot on/off?

- Date & time setting

- PXE/network boot option visible?

- BIOS supervisor password status (enabled/disabled)

## 🖥️ Section 3: Hardware Identification

Physically inspect your machine. Identify and write down the following components.
 When finished:

- Call Steve to check your answers.

- Once approved, grade one other student and sign off on their sheet.

### Hardware to Identify:

- CPU type (Intel/AMD + socket or model if you can)

- RAM type (DDR3, DDR4, or DDR5)

- Number of RAM sticks installed

- Storage type (HDD or SSD?)

- SATA or NVMe?

- Power supply rating (in watts)

- Number of fans (case + CPU)

- Number of USB ports (front + back)

- Expansion slots visible (PCIe x1/x16?)

## 🧾 Peer Sign-Off Sheet

| Section | Graded By (Initials) | Time Completed |
| --- | --- | --- |
| BIOS Hunt |  |  |
| Hardware ID |  |  |

## 📣 Rules

- You must complete all three sections to be marked present.

- You must grade one other student before leaving.