Lab: Transit Firewall and Router Policy with nftables

Debian-focused | forwarding policy | containerlab-friendly

Before you start This is a router and transit firewall lab. You will write rules on the firewall container, not on Inside1 or Inside2. The whole point is to control forwarded traffic.

# Goal

Use nftables on a Linux router to control traffic moving between networks. By the end, you will default-drop forwarded traffic, restore inside-to-outside access with stateful rules, and allow only web traffic from Inside1 to the DMZ.

# Time

One 2-hour class.

# Tools and prereqs

Firewall is a Debian container or VM with IP forwarding already enabled.

Inside1, Inside2, dmzweb, and outsideweb already have working IP configuration and default gateways.

The DMZ and outside machines have a web service running.

Commands should be run as root or with sudo on the firewall.

# Lab network used in this exercise

| Role | Host | Address / Notes |
| --- | --- | --- |
| Router / firewall | firewall | eth0 10.0.0.1 ¦ eth1 172.16.1.1 ¦ eth2 200.100.10.254 |
| Inside client | Inside1 | 10.0.0.10/24 |
| Inside client | Inside2 | 10.0.0.11/24 |
| DMZ web server | dmzweb | 172.16.1.8/24 |
| Outside web server | outsideweb | 200.100.10.1/24 |

# Deliverables

Screenshot 1: forwarded traffic broken by a default-drop forward policy.

Screenshot 2: Inside1 restored to the outside network using stateful forward rules.

Screenshot 3: Inside1 can open HTTP to the DMZ web server but cannot ping it.

Screenshot 4: Inside2 is blocked from the DMZ and the firewall shows matching counters.

# Part 1: Confirm the baseline

From Inside1, confirm that routing already works before you filter anything. Keep the output so you can compare the before and after state.

ping -c 2 172.16.1.8 ping -c 2 200.100.10.1 wget -O- http://172.16.1.8 wget -O- http://200.100.10.1

Check At this point Inside1 should be able to reach both networks. If baseline routing is broken, do not start the firewall portion yet.

# Part 2: Drop forwarded traffic by default

Load the smallest possible transit ruleset first. This will break forwarded traffic on purpose so you can see what the forward chain actually controls.

cat <<'EOF' ¦ sudo tee /root/transitfw.nft flush ruleset table inet transitfw { chain forward { type filter hook forward priority 0; policy drop; } } EOF sudo nft -f /root/transitfw.nft sudo nft list ruleset

Now go back to Inside1 and test again.

ping -c 2 200.100.10.1 wget -O- http://200.100.10.1

Capture now - Screenshot 1 Show that forwarded traffic from Inside1 is now failing, and include the firewall forward chain with policy drop.

# Part 3: Restore inside to outside traffic with state

A router firewall usually needs two ideas working together: a rule for the new outbound flow, and a rule that allows the reply traffic back in because it is part of an established connection.

cat <<'EOF' ¦ sudo tee /root/transitfw.nft flush ruleset table inet transitfw { chain forward { type filter hook forward priority 0; policy drop; ct state established,related counter accept ip saddr 10.0.0.0/24 ip daddr 200.100.10.0/24 counter accept } } EOF sudo nft -f /root/transitfw.nft sudo nft list ruleset

Test again from Inside1. The outside host should work now, but the DMZ should still be blocked because you have not allowed it yet.

ping -c 2 200.100.10.1 wget -O- http://200.100.10.1 wget -O- http://172.16.1.8

Capture now - Screenshot 2 Show successful access from Inside1 to 200.100.10.1 and include the firewall ruleset with the established,related rule plus the inside-to-outside rule.

# Part 4: Allow only web from Inside1 to the DMZ

Now turn this into an actual zone policy. Inside1 should be able to open HTTP to the DMZ web server, but ping should still fail because you only allowed TCP port 80. Inside2 should be blocked from the DMZ entirely.

cat <<'EOF' ¦ sudo tee /root/transitfw.nft flush ruleset table inet transitfw { chain forward { type filter hook forward priority 0; policy drop; ct state established,related counter accept ip saddr 10.0.0.0/24 ip daddr 200.100.10.0/24 counter accept ip saddr 10.0.0.10 ip daddr 172.16.1.8 tcp dport 80 counter accept ip saddr 10.0.0.11 ip daddr 172.16.1.0/24 counter drop } } EOF sudo nft -f /root/transitfw.nft sudo nft list ruleset

From Inside1, test the DMZ web server and then test ICMP to the same target.

wget -O- http://172.16.1.8 ping -c 2 172.16.1.8

Capture now - Screenshot 3 Show successful HTTP from Inside1 to 172.16.1.8 and failed ping to the same host. Keep the output readable.

# Part 5: Prove Inside2 is blocked from the DMZ

Move to Inside2 and try to open the same DMZ website. Then check the counters on the firewall so you can prove the block happened for the right reason.

wget -O- http://172.16.1.8

sudo nft list chain inet transitfw forward

Capture now - Screenshot 4 Show the failed HTTP attempt from Inside2 and the firewall forward chain with counters or rule hits that prove the traffic matched your policy.

# Discussion and cleanup

Why do these rules belong in forward and not output?

Why did Inside1 need an allow rule and an established,related rule to talk to the outside?

Why does HTTP to the DMZ work for Inside1 while ping to the same host still fails?

When you are finished, leave the final policy in place until your instructor checks it, or flush the ruleset if you are asked to reset the firewall for the next group.

sudo nft flush ruleset