Sci-Fi Serial Numbers (MAC Addresses):

- Star Trek:

  - USS-ENTERPRISE-NCC-1701-D

  - DATA-N-SOONG-11001001

  - BORG-QUEEN-0001C101101

  - Q-CONTINUUM-CHAOS4EVR

  - KLINGON-D7-KLAGH-BIK-ROJ

- Star Wars:

  - MILL-FALCON-YT1300-F987

  - R2D2-C3PO-BFF4E10481B

  - BB8-SPHERO-ROUNDANDROLL

  - DEATH-STAR-SUP3RWEAP0N

  - XWING-T65B-RED5-LUKE

- Other Sci-Fi:

  - TARDIS-BBC-TIMELORD-01

  - HAL9000-DISCOVERY-2001

  - DELOREAN-DMC12-BTTF1985

  - WALL-E-BN-L-LOVEEVE-700

  - IRON-GIANT-VIN-9999-A113

Street Addresses (IP Addresses):

- 01000010 Binary Boulevard (For those who speak fluent computer)

- 500 Internal Error Court (Where software meltdowns are a daily occurrence)

- 255.255.255.0 Subnet Circle (Home to the ultimate network nerds)

- 127.0.0.1 Home Sweet Home (The classic localhost address)

- 418 Coffee Break Place ("I'm a teapot" error inspiration, with a caffeine twist)

- 1600 Pennsylvania Ave

- 1337 Hax0r Way (leetspeak for "hacker")

- 1 Error Drive (perfect for those frustrating computer moments)

- 42 Universe Lane (a nod to "The Hitchhiker's Guide to the Galaxy")

- 604 Teapot Avenue ("I'm a teapot" error code reference)

- 999 Glitch Street (because technology isn't always perfect)

### Simplified Packet Relay Race

#### Overview:

Students act as network nodes transferring a "packet" of data from a source to a destination based on MAC and IP addresses. The goal is to correctly pass the packet along the shortest path, just as data would be routed through a network.

#### Setup:

- Arrange students in a configuration that simulates a network topology (e.g., circle, mesh, or line).

- Each student is assigned a fictional MAC address and IP address from the lists provided.

- Print or write these addresses on cards for each student to hold.

#### Materials:

- "Packets": Small envelopes or pieces of paper.

- Inside each packet:

  - Source MAC Address (e.g., USS-ENTERPRISE-NCC-1701-D)

  - Destination MAC Address (from the list provided to another student)

  - Source IP Address (e.g., 42 Universe Lane)

  - Destination IP Address (matching another student's address)

#### Instructions:

- Start the Game:

  - The instructor acts as the "DNS Server" and selects the starting student (source) and provides them with the packet.

  - The instructor announces the destination IP address to everyone.

- Finding the Path:

  - The student with the packet (source node) begins by reading the destination MAC address aloud.

  - The student with the matching MAC address responds and becomes the next node in the path.

  - Simultaneously, they read the destination IP address to ensure they are part of the correct route (like a double-check).

- Passing the Packet:

  - The packet is physically passed to the student who responded.

  - This continues, with each "node" announcing and passing until the packet reaches the student with the destination IP address.

- Ensuring Accuracy:

  - To add an element of error-checking, the student who is the final recipient must confirm both the MAC and IP addresses before declaring the packet successfully delivered.

- The End Game:

  - Time each round or make it a race with multiple packets starting from different sources at the same time.

  - Discuss the most efficient path for each packet and why it's important in networking.

#### Learning Points:

- How MAC addresses operate at Layer 2 (Data Link Layer) for physical addressing.

- How IP addresses operate at Layer 3 (Network Layer) for logical addressing.

- The importance of address resolution and path selection in data transfer.

### Simplified ARP Role-Play

#### Objective:

Students will role-play the ARP process to understand how network devices obtain the MAC address corresponding to a known IP address in a local network segment.

#### Setup:

- Each student is given a card with a fictional MAC address and IP address from the list provided.

- One student is designated as the “Sender,” who needs to communicate with a “Receiver,” but only knows the Receiver’s IP address, not their MAC address.

- If possible, arrange the seating to mimic a local area network (LAN) layout, where all devices are connected to a central switch or hub.

#### Materials:

- Cards with fictional MAC and IP addresses for each student.

- One “ARP Request” card that has an IP address but no MAC address.

#### Instructions:

- ARP Request Broadcast:

  - The Sender holds up the ARP Request card and announces to the class, “Who has [Receiver’s IP address]? Please reply with your MAC address!”

- Network Response:

  - All students check their cards. The student with the matching IP address is the Receiver.

  - The Receiver stands up and announces, “ARP Reply: [Receiver’s IP address] is at [Receiver’s MAC address]!”

- Completing the ARP Process:

  - The Sender notes down (or remembers) the Receiver’s MAC address for future communication.

  - The ARP Request card can be physically passed to the Receiver to signify the completion of the ARP exchange.